Indian Wells Valley Groundwater Basin

Groundwater Sustainability Plan

Section 3- bASIN SETTING

review Draft

June 14, 2019

Table of Contents

Section 3: Basin Setting	3-4

3.1	Introduction	3-4

3.2	History of Water Use in the Indian Wells Valley	3-5

3.3	Hydrogeologic Conceptual Model	3-7

3.3.1 Soils	3-8

3.3.2 Geology and Hydrogeology	3-9

3.3.3 Hydrology	3-11

3.3.4 Water Budget and Overdraft Conditions	3-14

3.3.5 Sustainable Yield	3-21

3.4	Current and Historical Groundwater Conditions and Hydrology	3-22

3.4.1 Seawater Intrusion Conditions	3-23

3.4.2 Groundwater Quality Conditions	3-23

3.4.3 Land Subsidence Conditions	3-25

3.4.4 Identification of Interconnected Surface Water Systems	3-25

3.4.5 Identification of Groundwater-Dependent Ecosystems (GDEs)	3-26

3.5	Numerical Groundwater Model	3-26

3.5.1 Initial Model Development	3-26

3.5.2 Flow Model Review and Recalibration	3-28

3.5.3 Baseline Conditions	3-31

3.6	Existing Monitoring Network and Evaluation	3-31

3.6.1 Existing Monitoring Network	3-31

3.6.2 Monitoring Network Assessment and Data Gap Evaluation	3-31

3.7	References	3-33

# Basin Setting

## Introduction

The GSP Emergency Regulations Guide “GSP Guide” (DWR 2016a) calls for development of a Basin Setting that includes a description of the static physical characteristics of the basin and the dynamic groundwater and water budget conditions.  The descriptive hydrogeologic conceptual model of the IWV will be used to describe basin setting static conditions.  The hydrogeologic conceptual model will provide stakeholders with an “understanding of the basin’s physical characteristics related to hydrology, land use, geology and geologic structure, water quality, principal aquifers, and principal aquitards…” (DWR 2016b). The hydrogeologic conceptual model will also provide an understanding of how “aquifers react to hydrologic stresses over time, and the interaction of surface water and groundwater systems within the basin” (DWR 2016a). The intent is for the hydrogeologic conceptual model to be used as an informational tool to facilitate a shared understanding among all parties regarding groundwater behavior and cause and effect relationships of actions affecting the IWV such as the implementation of projects and management actions.

The GSP Guide (DWR 2016a) also states that dynamic groundwater conditions should be described by historical and present-day groundwater conditions related to undesirable results, and that the GSP should include a description for conditions as of January 1, 2015.  Data gaps and data uncertainty that limit basin understanding or evaluation of GSP performance must be noted (DWR, 2016a).

In addition to the hydrogeologic conceptual model, the Basin Setting should also include a quantitative water budget that accounts for inflows and outflows.  Groundwater basins subject to critical overdraft, such as the IWVGB, must quantify the amount of overdraft.  Baseline conditions related to supply, demand, hydrology, and surface water supply reliability are established for the purpose of understanding future projected conditions and for development of management actions and projects.

In keeping with requirements in the GSP Guide, this section provides background and discussion of: 1) the numerical groundwater model of the IWVGB which has been developed by the Department of the Navy; 2) the hydrogeologic conceptual model; 3) the historical water budget; and 4) a summary of the current and historical groundwater conditions and hydrology.

## History of Water Use in the Indian Wells Valley

The first water use in the IWV correspond with the first habitation of the valley by Native Americans around 10,000 years ago. Evidence of prehistoric village sites in the IWV suggest a sizeable resident population. The region during that time was different than it is today, characterized by a cooler and wetter climate with numerous springs and streams.  The Owens River flowed through the valley, and China Lake was a large fresh-water lake. The valley was surrounded by conifer forests similar to the forests seen today along the Sierra Crest, in areas above Walker Pass (Harris, 2013a).

Approximately 7,000 years ago, the region’s climate started to warm and dry, continuously evolving into the Mojave Desert landscape of today. Research indicates the creosote bush, which is prevalent in the IWV today, had arrived around that time. The Owens River last flowed through the IWV around 2,500 years ago. As the warmer climate dried up many perennial water sources, the Native Americans in the IWV migrated into the canyons of the Sierra Nevada to be nearer to stable water supplies (Harris, 2013a).

The first Europeans arrived in the valley in the first half of the 1800s, consisting of trappers, adventurers and explorers. Joseph Walker passed through in the early 1830s, and then returned with the Fremont party in 1845, which included famous explorers Kit Carson, Richard Owens, and Ed Kern. These early European arrivals in the IWV were transient, with many of these parties making use of perennial Indian Well Spring, where the lodge and brewery along Highway 14 is today, as they passed through the IWV. (See Figure 2-2 for the location of IWV landmarks.) Travelers during the California Gold Rush, the infamous Death Valley Party, and the Whitney Geological Party are all thought to have utilized perennial springs during their travels (Harris, 2013a).

The second half of the 19th century brought more permanent settlements along the Eastern Sierra with the development of mining operations.  Passenger and freight routes were developed to transport silver ore. Given the arid climate, fresh water was a valuable commodity and exclusively came from surface flows at springs and streams. Sites such as Coyote Holes near Freeman Junction, Indian Wells, and Little Lake became noted water stops.  Mineral deposits were mined along Searles Lake and in the Panamint Range, creating additional freight stage coach routes running between water sources. Basque sheep herders also arrived in the IWV during the 1860s, and drove their sheep though the valley every year in route to their summer ranges north of Bishop (Harris, 2013a).

The composition of the community and water landscape changed toward the end of the 1800s when a large gold discovery in Randsburg attracted thousands of people into IWV and surrounding regions.  Mines were developed along the south edge of the Indian Wells Valley, and many operations required groundwater to supply the milling needs.  The infrastructure became part of the support for the future construction of the Los Angeles Aqueduct that connects water from the Owens Valley to the City of Los Angeles.  These projects and operations caused an influx of people into the valley for work, which led to the establishment of the communities of Inyokern and Brown (Harris, 2013a).  As development in the late 1800s continued, it became necessary to develop groundwater supplies to support the increased population, infrastructure, and operations in the IWV.

The beginning of the 20th century saw a migration of people into the IWV to homestead and farm. As a significant number of government land titles were issued, agriculture land use increased, which resulted in an estimated 1,000 acre-feet (AF) of groundwater use to support local farms by 1920 (Harris, 2013a). Typically, alfalfa was grown in the IWV to feed dairy cattle, which were raised to produce cheese for export (Harris, 2013a).

Industrial groundwater use also greatly increased during the first decades of the 20th century. The American Trona Corporation constructed a pipeline out of IWV to the Westend Chemical Plant in Searles Valley in 1918.  Around 1930, an additional pipeline was constructed to export water to Searles Valley for use at the American Potash & Chemical (previously known as American Trona Corporation) plant in Trona (Harris, 2013a). After multiple acquisitions, the mining company in Trona became Searles Valley Minerals Incorporated in 2004, as it is known today. Searles Valley Minerals Inc. continues to be largest industrial user and only exporter of water from the IWVGB.

World War II brought permanent changes to the IWV by introducing the U.S. Navy to the region. In 1943, the Navy began development of an Ordnance Test Station, which included construction of hundreds of industrial and residential buildings, roads, and other necessary infrastructure.  As development by the Navy continued, it was necessary to drill more groundwater wells to supply the increased water demands. Most of the IWV’s new permanent residents were associated with the naval operations and lived on Navy property during the 1940s and 1950s (Harris, 2013b).  The growth of the naval operations led to the incorporation of the City of Ridgecrest in 1963. In the 1970s and 1980s, Navy personnel began shifting to off-Base housing in the City of Ridgecrest, which transferred water demands from the U.S. Navy service area to the Indian Wells Valley Water District (Harris, 2013b).

Water use in the IWV over the past 50 years has been documented, first by the U.S. Bureau of Reclamation and then by the Cooperative Group since 1975 (Harris, 2013b).  Water use data from 1975 records show total groundwater production of 15,980 AF, with the Navy accounting for 31% of pumping (Table 3-1).  Data from the Cooperative Group (see Appendix 3-A) suggests peak water usage in the IWV at approximately 29,500 AF in 2007. After significant conservation efforts, primarily by the Navy and the Water District, groundwater production in the IWV has been reduced to approximately 25,300 AF in 2015, with a shift towards agriculture use. The water use category distribution in 2015 had changed significantly from 1975 as shown by the following tabulated distribution.

Table 3-1  Historical Pumping Distribution by Water Use

| Water Use | 1975 15,980 AF | 2007 29,500 AF |
| --- | --- | --- |
| Agriculture | 22% | 50% |
| Industrial | 17% | 10% |
| City/Municipal/Domestic | 29% | 34% |
| U.S. Navy | 31% | 6% |

## Hydrogeologic Conceptual Model

The geology and hydrogeology of IWVGB have been studied since the early 1900’s.  The California Conservation Commission (Lee, 1912) wrote one of the first reports to estimate the available groundwater resources within the basin.  Multiple studies were completed by the United States Geologic Survey (USGS) and DWR to delineate the geologic and hydraulic features (Moyle, 1963; Kunkel and Chase, 1969; Dutcher and Moyle, 1973; Berenbrock, 1987).  Further geologic work was recently completed by the USBR in 1993, the IWVCGTAC (TriEcoTt, 2013) under an AB303 Grant, and SkyTEM geophysical survey under Prop1 Funding (Thorn, et al., 2019).  Recent comprehensive reports completed by Kern County (Todd, 2014) and the Navy (DRI: McGraw et al., 2016; Garner et al, 2017) have provided historical trends in groundwater levels and summarized the current understanding of the occurrence and movement of groundwater.  Following is a general description of the hydrogeologic conceptual model that has been a foundation for the numerical groundwater model and development of the GSP water budget analysis.

The IWV hydrographic basin is bounded by the Sierra Nevada Mountains to the west, the Coso Range to the north, the Argus Range to the east, and the El Paso Mountains to the south.  The mountain slopes dip steeply to the valley floor that in turn slopes gently to China Lake, which is usually a dry playa.  The location map (Figure 3-1) shows the groundwater basin boundary (also known as the GSA boundary), the watershed extents, and land ownership for reference to this discussion.

The hydrogeologic conceptual model shown in Figure 3-2 provides an illustration of the general structure of the basin with the primary recharge in and discharge out of the groundwater aquifer.  The natural recharge occurs along the mountainfront, in the mountain stream alluvium, and as subflow from Rose Valley (near Little Lake).  The main discharge of groundwater occurs from pumping wells, evapotranspiration (ET) at the playa, and estimated subflow to Salt Wells Valley.  The general flow direction of the groundwater system is from the mountains (recharge area) towards the playa (discharge area).  This section describes the soils, geology, hydrogeology, and hydrology of the basin, followed by a description of the historical water budget and current understanding of the sustainable yield for IWVGB.

### Soils

A review of the Natural Resource Conservation Service (NRCS) Soil Survey Geographic Database (SSURGO) revealed that soils data for the majority of the IWV are not available, as shown on Figure 3-3.  Data for two soil types present in the El Paso area are available, however: Aridisols are typically too dry for the growth of mesophytic plants and regularly accumulate easily leached minerals from humid environments; Entisols occur in environments where erosion or deposition rates are faster than soil development rates and typically occur on dunes, steep slopes, and flood plains (USDA NRCA, 2019).  Based on climate and the map of dominant soil orders of the United States (NCRS, 1998), it can be assumed that the soil types described above are dominant within the IWV.

### Geology and Hydrogeology

Indian Wells Valley lies within the northern portion of the Eastern California Shear Zone (ECSZ) or the southern Walker Lane Belt, in a transitional zone of east-west extensional faulting of the Great Basin Province and the dominant right-lateral strike-slip faulting common to the Sierra Nevada Mountains (TriEcoTt, 2013).  From the Late Miocene through the Pliocene, IWV was down-faulted along the Sierra Nevada frontal fault resulting in the structural half-graben of present-day (Monastero et. al., 2002; Kunkel & Chase, 1969).   Based on historical groundwater levels, the Little Lake fault zone (LLFZ) and the El Paso fault (EPF) (mapped by Garner et al., 2017) have been shown to impede the movement of groundwater within the basin.   Figures 3-4a and 3-4b show the surficial geology in the IWVGB and Figures 3-5a and 3-5b show two cross-sections.

During the Pleistocene epoch (1.8 million to 12,000 years ago), the region was much wetter and surrounding basins that are currently isolated (for example:  Rose Valley, Owens Valley, Indian Wells Valley, Salt Wells Valley, and Searles Valley) were all connected via the Owens River.  The active movement of alluvial fan material off the Sierra Nevada and the reworking of basin sediments by the Owens River resulted in a complex distribution of well-sorted to poorly sorted deposits in the IWV Basin (Dutcher and Moyle, 1973).  Isolated basins were sequestered through subsequent faulting, volcanic activity, and deposition of lacustrine deposits, contributing to a decline in flow in the Owens River (Kunkel and Chase, 1969).

Because of the low permeability bedrock of the surrounding mountain ranges, the groundwater basin receives no significant recharge from beyond the topographic divides of the watershed boundary, except for subflow from Rose Valley.  Large alluvial fan complexes stemming primarily from the Sierra Nevada, and to a lesser degree from the other mountain ranges, allow groundwater flow into the basin as mountain block recharge (Dutcher and Moyle, 1973).  Dutcher and Moyle (1973) estimated that the depth of water-bearing alluvial deposits could locally reach to 2,000-feet.  These sediments consist mainly of gravel and sand in alluvial fans near the mountains and grade to silt and clay beneath the playa.

Lithologic units are taken from Berenbrock and Martin (1991) who based their work on units mapped by Von Huene (1960), Zbur (1963), and Kunkel and Chase (1969).  The geologic units include consolidated bedrock and unconsolidated sediments.  Bedrock is typically low in permeability and porosity unless it is highly fractured.  These rocks include the tertiary igneous and metamorphic rocks in the surrounding mountains, as well as the Miocene basalts near the El Paso Mountains.

For the GSP, the groundwater depletion that is of concern in IWVGB is from the unconsolidated alluvial deposits.  These water-bearing sediments store and transmit water and are divided into the following hydrostratigraphic features that are important for analyzing sustainability criteria and groundwater budgets.  These unconsolidated units are mapped across the project site as alluvium, lacustrine, and playa.  Berenbrock and Martin (1991) describe these units as follows:

- Alluvium consists of moderately to well-sorted gravel, sand, silt, and clay of Pleistocene and Holocene age (12,000 years ago to present) and is considered to have a high permeability. The percentage of silt and clay increases toward the central portion of China Lake, which reduces permeability. These deposits include both older and younger alluvial deposits, alluvial fans, and elevated pediment veneers and stream terrace deposits. Alluvium extends across IWV and is thickest along the western and southern edges of the basin (Figures 3-5a and 3-5b).

- Lacustrine deposits were described by Kunkel and Chase (1969) as containing silt and silty clay of Pleistocene age and exhibiting low permeability. This unit lies above the alluvium and is interbedded with the alluvial deposits in the central portion of the basin.

- Playa deposits of low permeability are of Holocene age and contain silt and clay with an occasional sand lens.

There are two principal aquifer units defined by Kunkel and Chase (1969).  The shallow aquifer contains coarse sediments near the Sierra Nevada with increased interbedded silts and clays towards the center of the basin associated with the lacustrine and  China Lake playa deposits, .  The best quality of water is at shallow to medium depths in the western part of the valley, closer to the Sierra Nevada (Dutcher and Moyle, 1973).  The deeper aquifer is also composed of gravel, sand, silt and clay.  It is connected to the shallow aquifer in the west and southwest of the basin.  Existing CASGEM multi-level monitoring wells show semi-confined artesian conditions within the deeper aquifer where it occurs beneath the lacustrine and playa fine-grained sediments.

Historical well drilling and aquifer testing results were compiled for background data of hydraulic conductivity throughout the IWVGB.  Hydraulic conductivity has been determined to range from < 5 feet per day (ft/day) to > 50 ft/day based on slug and aquifer testing.  The distribution of hydraulic conductivity throughout the IWVGB is shown in Figure 3-6 and the testing results are shown in Appendix 3-B.  In general, the shallow and deep aquifer zones are associated with higher hydraulic conductivity.  Although there are no historical aquifer tests conducted within the lacustrine/playa deposits, it is expected that these units are associated with lower hydraulic conductivity that impedes flow.

### Hydrology

#### Climate and Precipitation

IWVGB is part of the Mojave Desert and has an arid, high desert climate characterized by hot summers, cold winters, and little precipitation. The basin is bounded by mountains to the north, south, and west, which drain internally to the playa. Summer high temperatures on the playa are typically greater than 100 degrees Fahrenheit (°F) and winter lows are typically in the 20s and 30s °F.

Average annual precipitation on the playa is about 4 inches per year; snowfall occurs in December and January, with an average of 1.3 inches per year (WRCC, 2018c; PRISM, 2012). The mountain areas receive more precipitation than the playa and are the primary source of recharge for the groundwater basin. The average annual precipitation for the IWVGB and the mountain areas ranges from about 4 inches per year up to about 20 inches per year (PRISM, 2012) (Figure 3-7). The annual precipitation and cumulative departure from mean at two stations near the IWV is shown in Figure 3-8.

With high temperatures and limited precipitation, the IWVGB has high ET rates. Average annual evaporation from a shallow water body in the playa is about 80 inches per year (Farnsworth et al, 1982). Evaporation in the mountain areas is less than on the basin floor.

#### Streamflow and Mountain-Front Recharge

Streamflow gaging stations in and near IWVGB are shown on the map in Figure 3-7. The USGS historically collected data at several tributaries. The stations were active in the 1960s and 1970s and have five to ten years of observations. The daily hydrograph at Ninemile Creek (USGS gage 10264878), which drains a 10.4-square mile area of the Sierra Nevada, is shown in Figure 3-9.  Daily discharge at the gage was flashy and sporadic, with short peaks and zero flow between storms. In two years (1964 and 1968), flow was nearly zero for the entire year.

Streamflow observations at Sand Canyon and Grapevine Canyon have been collected through a cooperative monitoring effort by IWVGA (Ribble and Haslebacher, 2000; Haslebacher, 2018). The gage at Sand Canyon has a drainage area of about 18 square miles. Flow is measured with a rectangular weir and datalogger, with records dating back to 1999. Annual gaged streamflow at Sand Canyon is given in Table 3-2 and ranges from 69 AFY to 3,783 AFY. The average annual streamflow is 647 AFY and the median is 209 AFY. Grapevine Canyon, with a drainage area of about 10 square miles, is gaged with a v-notch weir and datalogger, with records dating back to 1997. As shown in Table 3-2, annual flows at Grapevine Canyon range from 4 AF to 2,275 AF, with average annual streamflow of 237 AFY and a median of 90 AFY. At both gages, flow during wet years, i.e. 1998 and 2005, skews the average toward high flows.

Table 3-2. Annual Gaged Streamflow at Sand Canyon and Grapevine Canyon.

| Calendar Year | Sand Canyon Gaged Streamflow (AF) | Grapevine Canyon Gaged Streamflow (AF) |
| --- | --- | --- |
| 1997 | --1 | 49 |
| 1998 | --1 | 2,275 |
| 1999 | 200 | 394 |
| 2000 | 217 | 95 |
| 2001 | 150 | 43 |
| 2002 | 69 | 19 |
| 2003 | 329 | 16 |
| 2004 | 102 | 4 |
| 2005 | 3,783 | 1,160 |
| 2006 | 585 | 193 |
| 2007 | 171 | 35 |
| 2008 | --2 | --2 |
| 2009 | --2 | --2 |
| 2010 | --2 | --2 |
| 2011 | 184 | --2 |
| 2012 | 286 | 90 |
| 2013 | --2 | --2 |
| 2014 | --2 | --2 |
| 2015 | --2 | --2 |
| 2016 | --2 | --2 |
| 2017 | 1,683 | 563 |
| 2018* | 86 | 32 |
| Complete Years | 12 | 13 |
| Average | 647 | 237 |
| Median | 209 | 90 |
| Source: Haslebacher, 2018 1. No data available; gage not yet active. 2. No data available; measurements were not reported for these years. *Total through March 2018; value for 2018 not included in statistics. |  |  |

Mountain block recharge in IWVGB is believed to be the dominant source of inflow to the groundwater basin. In 2014, Todd Engineers prepared a study that reviewed previous recharge studies and made new estimates (Todd Engineers, 2014). In 2016, DRI conducted a comprehensive review of recharge estimates for the basin (McGraw et al, 2016). They reviewed fourteen previous studies and then updated the recharge estimates using an empirical relationship between precipitation and groundwater recharge. The average annual recharge developed by DRI is 7,650 AF per year (McGraw et al, 2016; Garner et al, 2017). The recharge zones identified by DRI are shown in Figure 3-10. The total area of recharge is about 770 square miles. The area and estimated annual recharge in each zone are shown in Table 3-3.

Table 3-3. Recharge Zones and Estimated Annual Recharge.

| Recharge Zone | Area (sq miles) | Annual Recharge1 (AFY) |
| --- | --- | --- |
| Rose Valley | 193 | 2,400 |
| Sierra Nevada N | 116 | 1,500 |
| Sierra Nevada S | 101 | 2,100 |
| El Paso | 56 | 50 |
| Argus and Coso | 302 | 1,600 |
| Total | 768 | 7,650 |
| 1 Recharge areas and annual volumes as developed by DRI (McGraw et al, 2016; Garner et al, 2017) |  |  |

Surface water in Little Lake, located in Rose Valley, is thought to infiltrate into groundwater in IWVGB. Estimates of losses from Little Lake are included in the annual recharge amount in Table 3-3 (McGraw et al, 2016). Other than Little Lake, there are no significant interconnected surface water systems which interact with groundwater. Generally, streams in the valley are ephemeral and recharge occurs as mountain block recharge.

IWVGB has many natural springs, shown in Figure 3-10, generally located in the mountain areas. Each point represents a spring or seep as mapped by the USGS in the National Hydrographic Dataset (NHD) (USGS, 2019). Springs in IWVGB have historically been used for water supply, cattle, and wildlife. Spring water quality and geochemistry have been studied previously (Stoner et al, 1995; Houghton HydroGeo-Logic, 1996).  The water quality of the spring/surface water samples collected is relatively good.  Although TDS concentrations range from 199 mg/L to 1,300 mg/L, the average TDS concentration for these samples is 533 mg/L.  The majority of samples collected from springs/surface water sources fall within the recommended TDS Secondary MCL and the upper TDS Secondary MCL for potable water, 500 mg/l and 1,000 mg/L, respectively. Data are included in Appendix 3-C.

### Water Budget and Overdraft Conditions

A water budget is an accounting tool that quantifies inflows (sources) and outflows (sinks) occurring within a groundwater basin (or specified management area) using the following equation:

Inflows – Outflows = Change in Storage

The water budget is a key component of overall understanding of the IWVGB and contributes to developing the following GSP elements:

- Identifying data gaps

- Evaluating monitoring requirements

- Evaluating potential projects and management actions

- Estimating the sustainable yield

- Evaluating undesirable impacts

- Informing water management decision-making

#### Water Budget Elements

The elements contributing to the IWVGB water budget include recharge, groundwater pumping, ET, and interbasin flow.

Recharge

Mountain front recharge, predominantly from the Sierra Nevada Mountains, is the primary inflow into the IWVGB. The Coso Range, Argus Range, and the El Paso Mountains contribute to the natural recharge to a lesser degree. Numerous basin studies have been prepared to estimate natural recharge to the IWVGB with various methods and with varying results. The methodology used in selected previous studies is described in McGraw et al. (2016). The historical natural recharge estimates from selected previous recharge studies are shown below in Table 3-4. In addition, the USGS is currently revising an existing Basin Characterization Model to refine the recharge estimates in the IWV.

Table 3-4: Natural Recharge Estimates from Selected Recharge Studies (AFY).

| Recharge Study | Natural Recharge Estimate (AFY) |
| --- | --- |
| Brown and Caldwell (2009) | 8,900 |
| Epstein et al. (2010) | 5,800 to 12,000 |
| Todd Engineers (2014) 1 | 6,100 to 8,900 |
| Desert Research Institute (McGraw et al. 2016) | 7,650 |

1 Excludes estimates of recharge from excess irrigation and distribution system leakage.

Groundwater Pumping

DRI developed a groundwater pumping database to represent historical pumping and to assist with making future pumping projections (McGraw et al., 2016). The database contains pumping from 1920 to 2013. The USGS provided the pumping estimates from 1920 to 1974 and the Cooperative Group provided the pumping estimates from 1975 to 2013 (McGraw et al., 2016). Pumping wells were assigned to one of the following water use categories:

- Private domestic

- Municipal

- Searles Valley Minerals (industrial)

- U.S Navy (NAWS China Lake)

- Agriculture

Well locations and use were cross-referenced and verified using published existing databases and aerial photographs. In situations where historical data for individual wells were not available, pumping rates were evenly distributed among appropriate wells within each water use category (McGraw et al., 2016).

The Cooperative Group has published annual production data dating back to 1975, organized by large users and primary categories. Groundwater production estimates from 1975 through 2016 as compiled by the Cooperative Group are provided in Appendix 3-A.  The location of all groundwater production wells in the IWV is shown in Figure 2-3.

Approximately 800 private domestic wells exist in the IWV outside of the City of Ridgecrest and the community of Inyokern.  These wells serve individual residences and typically pump around 1 AF per year with an approximate water use of 800 AF in 2015. Additionally, many private residences have formed small mutual water companies and co-ops with a single well serving multiple residences. Approximate water use for these mutual and co-ops is approximately 300 AF in 2015.

Since 1975, the Water District’s service area and population have expanded, resulting in corresponding increases in groundwater extractions; however, the Water District has implemented several demand management programs and conservation measures to bring their water use down to 7,050 AF in 2015, from a peak of approximately 9,200 AF in 2007. (See Section 2.6.4 for discussion of Water District conservation). Other municipal and domestic suppliers in the IWV include the City of Ridgecrest, which irrigates recreational parks and sports complexes, and the Inyokern Community Services District. Total domestic and municipal water use was 7,568 AF in 2015.

Since 1975, the reported industrial water use within the IWV by Searles Valley Minerals has remained fairly constant at approximately 2,600 AFY on average. Searles Valley Minerals primarily uses the exported groundwater for their solution mining operations; however, they also provide potable water to the small community of Trona that supports the Searles Valley Minerals operation plant.

The U.S. Navy uses groundwater to meet NAWS China Lake on-base water demands. Water uses on NAWS China Lake include potable water for office buildings, laboratories, residences, and schools. In 1975, the U.S. Navy reported their highest groundwater use at 5,000 AF. Over decades, as base personnel increasingly moved to off-base housing in Ridgecrest and after implementing of aggressive water conservation programs beginning in 2007, the U.S. Navy reduced their water use to 1,595 AF in 2015.

Agriculture water use has increased significantly in recent decades as new ranches, orchards, and farms have developed in the IWV. Alfalfa and pistachios are the largest crops by volume grown in the IWV, with olives, tomatoes, and other crops producing significantly less. Pistachios typically require a water application rate of approximately 5 feet per year, while alfalfa requires a higher rate of 7 feet per year or more (Todd, 2014; McGraw et al., 2016). Total agriculture water use in 2015 was over 12,000 AF, comprising almost 50% of the total water use that year. Unless restricted, agricultural use is expected to increase significantly.

Evapotranspiration

The ET that occurs in the China Lake Playa area is the primary natural discharge for the IWVGB. Prior to development of well fields around the 1920s, ET from the China Lake Playa was the predominant outflow from the IWVGB. Todd Engineers (2014) noted that estimates of ET have decreased over time; “[some] of the decrease is attributable to revised estimates of recharge based on model calibration, but most of it reflects the interception of playa outflow by wells.” Declines in water levels alter and reduce phreatophyte vegetation, reducing transpiration, and reduce bare surface evaporation rates. Vegetation changes have been assessed by comparing maps of the current vegetation distribution to the pre-development vegetation map of Lee (1912). The major difference is the addition of greasewood in areas north and east of the playa, and also in a small area to the southwest, where pickleweed and saltgrass occurred previously (McGraw et al., 2016). The pickleweed and saltgrass vegetation zone is associated with a shallower water table with a maximum ET rate of 5.7 ft/yr, and ET terminates when the water table is greater than 10 ft bgs. The greasewood unit that develops as water levels decline has a maximum ET rate of 2.4 ft/yr and a maximum rooting depth of 33 ft. Current bare playa evaporation rates have been estimated from data from an eddy covariance station at the south end of China Lake playa and suggest annual ET of 4.5 inches for the adjacent bare playa area (McGraw et al., 2016). This measurement removes rainy days from the calculation, but bare playa groundwater ET could be as low as 2.4 inches per year, if ET is affected for several days after significant precipitation events.  Current overall ET loss from IWVGB is estimated at 4,850 AFY.

Interbasin Flow

Previously studies on the IWVGB have primarily considered the IWVGB to be a closed basin with little to no subsurface outflow to Salt Wells Valley. Nonetheless, DRI concluded “the absence of a large accumulation of salinity in Indian Wells Valley suggests that the basin may not be hydrologically closed” (McGraw et al., 2016). Furthermore, DRI noted that water levels within IWV “are higher than in Salt Wells Valley, which indicates that interbasin groundwater flow is a possibility given large enough transmissivities” (McGraw et al., 2016). DRI performed a hydraulic analysis of the Salt Wells Valley and concluded that it is possible that approximately 50 AFY of the groundwater flow in the Salt Wells Valley originates as underflow from the IWV as compared to mountain front recharge from the Argus Range.

#### Historical Water Budgets

As discussed previously in Section 3.2, groundwater extractions began around the 1920s when farmers and industrial users began settling into the IWV. Prior to the development of well fields, it is assumed the IWVGB was in hydrostatic equilibrium, with inflows in balance with outflows. The estimated water budget during pre-development conditions (i.e. prior to the 1920s), originally developed by DRI (McGraw et al., 2016), is shown below in Table 3-5.

Table 3-5. Steady-State Water Budget (Pre-Development Conditions).

| Water Budget Element | Estimated Volume (AFY) |
| --- | --- |
| Inflows |  |
| Mountain Front Recharge |  |
| Sierra Nevada North | 1,500 |
| Sierra Nevada South | 2,100 |
| Rose Valley | 2,400 |
| Coso/Argus Ranges | 1,600 |
| El Paso | 50 |
| Total Inflow | 7,650 |
| Outflows |  |
| ET | 7,450 |
| Interbasin Subsurface Flow | 200 |
| Groundwater Extractions | 0 |
| Total Outflow | 7,650 |
| Change of Groundwater in Storage | 0 |

Source: Adapted from Table 6, McGraw et al. (2016).

As industrial, agricultural, and residential development expanded beginning in the 1920s, groundwater extractions increased which reduced the ET occurring at China Lake Playa and reduced subsurface flow to the Salt Wells Valley.  The historical average post 1920 estimated water budget since IWV was developed is shown below in Table 3-6.

Table 3-6. Historical Water Budget (1922 to 2016)

| Water Budget Element | Estimated Volume (AFY) |
| --- | --- |
| Inflows |  |
| Mountain Front Recharge | 7,650 |
| Total Inflow | 7,650 |
| Outflows |  |
| ET | 6,580 |
| Interbasin Subsurface Flow | 60 |
| Groundwater Extractions | 15,240 |
| Total Outflow | 21,880 |
| Change of Groundwater in Storage | -14,230 |

#### Current Water Budget

In more recent years, agricultural water demands have increased resulting in higher groundwater extractions compared to the long-term average and reductions in the ET occurring at China Lake Playa and subsurface flow to the Salt Wells Valley.  The current average estimated water budget for IWV is defined as the years 2011 to 2015 and is shown below in Table 3-7.

Table 3-7. Current Water Budget (2011 to 2015).

| Water Budget Element | Estimated Volume (AFY) |
| --- | --- |
| Inflows |  |
| Mountain Front Recharge | 7,650 1 |
| Total Inflow | 7,650 |
| Outflows |  |
| ET | 4,850 |
| Interbasin Subsurface Flow | 50 |
| Groundwater Extractions | 27,740 |
| Total Outflow | 32,640 |
| Change of Groundwater in Storage | -24,990 |

1 Long term historical average

#### Overdraft Conditions

A water budget is defined by the difference between inflows and outflows (see Section 3.3.5). Overdraft occurs when outflows exceed inflows, and there is a loss of water in storage. It is well documented that IWV has been in overdraft since the 1960s (Dutcher and Moyle, 1973). Currently (2011 to 2015), outflows are approximately four times the inflows into the IWV. The magnitude of the overdraft results in an average annual loss of storage of approximately 25,000 AFY (See Table 3-7 Current Water Budget). This loss of storage equates to decline of water levels at a rate of approximately 1-2 feet per year (Garner et al., 2017). Todd (2014) characterized the evidence of overdraft in the IWV by stating “[the] ubiquitous, long term and ongoing decline in water levels is the most definitive evidence of groundwater overdraft.” Hydrographs of wells taken throughout the IWV demonstrate significant prolonged drawdown. Groundwater elevation data including groundwater elevation contour maps and selected well hydrographs from wells distributed throughout the IWV are provided in Appendix 3-D.

According to California Water Code Section 12924, DWR is required to investigate groundwater extractions and recharge patterns within California’s groundwater basins and identify groundwater basins in critical conditions of overdraft.  DWR has determined the IWVGB meets their definition of critical overdraft defined in DWR Bulletin 118-80 (1980) as when “continuation of present water management practices would probably result in significant adverse overdraft-related environmental, social, or economic impacts.”

Consequences of prolonged overdraft, beyond the loss of groundwater in storage, include chronic lowering of groundwater levels, increased pumping costs, loss of well yields, water quality degradation, and land subsidence. These consequences of overdraft, characterized as significant and unreasonable, will be addressed and mitigated as applicable through implementation of this GSP in order to prevent further environmental, social, and economic impacts.

[Discussion of Groundwater in Storage not yet Completed]

### Sustainable Yield

DWR states that “SGMA requires local agencies to develop and implement GSPs that achieve sustainable groundwater management by implementing projects and management actions intended to ensure that the basin is operated within its sustainable yield by avoiding undesirable results” (DWR, 2016d). Consequently, the sustainable yield is a crucial and fundamental element for the development of implementation measures of the GSP. The results of the water balance analysis indicate a natural long-term average recharge of 7,650 AFY which is the value of the sustainable yield. This sustainable yield is used as the basis for further GSP analysis of water supply projects. During the preparation of 5-year periodic updates of the GSP, the sustainable yield will be reevaluated.

Given the state of critical overdraft in the IWV, the IWVGA must implement water supply and demand management projects and measures in order to operate within the sustainable yield and maintain the community. According to Todd (2014), “[the] magnitude of the overdraft indicates that recovery cannot be achieved with conservation or water recycling alone without jeopardizing beneficial uses.” The natural sustainable yield can be augmented with direct recharge projects (e.g. recycled water or imported water) in order to create an operating sustainable yield higher than the natural sustainable yield. It is a priority to implement water supply projects when feasible to lessen impacts to the community and current ways of life that would invariably be altered if significant and drastic water use reductions are required in order to reach the current natural sustainable yield.

## Current and Historical Groundwater Conditions and Hydrology

SGMA defines a “sustainability indicator” as an “effect caused by groundwater conditions occurring throughout the basin that, when significant and unreasonable, cause undesirable results” (GSP 2016). SGMA further identifies the following six sustainability indicators as those required to be addressed in a GSP:

- Chronic Lowering of Groundwater Levels

- Reduction of Groundwater Storage

- Seawater Intrusion

- Degraded Water Quality

- Land Subsidence

- Depletion of Interconnected Surface Water

The chronic lowering of groundwater levels and loss storage is previously discussed in Section 3.3.5.4. The remaining four sustainability indicators are discussed in the subsections below in the context of identifying the presence of these current conditions. SGMA requires Groundwater Dependent Ecosystems (GDEs), to be evaluated; while not a specific sustainability indicator, GDEs are discussed in a subsection below along with the sustainability indicators because they could be impacted by the chronic lowering of groundwater levels. The understanding of current conditions existing in the IWV will support evaluation of the sustainability indicators and will inform basin management decisions such as implementation of projects and management actions and establishment of criteria to track goals for achieving sustainability (See Section 4 and Section 5).

### Seawater Intrusion Conditions

The IWVGB is an inland basin, and as such, is not hydraulically connected to a sea or ocean. The City of Ridgecrest is over 100 miles from both the Pacific Ocean and the Salton Sea. Accordingly, seawater intrusion is not evaluated in this GSP and seawater intrusion will not be considered as a sustainability indicator for establishing sustainable management criteria (see Section 5).

### Groundwater Quality Conditions

Currently, groundwater in the IWVGB is generally of good quality; however, there are regions with poorer water quality due to high concentrations of Total Dissolved Solids (TDS) and arsenic. These constituents and resulting water quality conditions are discussed in the subsection below.

#### Total Dissolved Solids

TDS is a measure of all dissolved solids in water including organic and suspended particles. Sources of TDS in groundwater include interaction of groundwater with the minerals that comprise the aquifer matrix material. Over time, TDS will increase as more minerals in contact with groundwater dissolve.  In desert basins, evaporative enrichment is known to naturally increase TDS in groundwater. This process also occurs in plants, both in agricultural and natural systems.  Anthropogenic sources include synthetic fertilizers, manure, wastewater treatment facilities, and septic effluent. Repeated irrigation is also a known cause of elevated TDS, as minerals concentrate in the soil column with repeated evaporation. These increased concentrations can then be mobilized into the underlying groundwater table.  The concentration of TDS in groundwater within the IWVGB has been studied and documented for several decades.  The California Water Board regulates TDS as a Secondary Maximum Contaminant Level (SMCL), a contaminant affecting the taste and odor of drinking water (CA Water Boards 2019).  There is no Primary MCL for TDS.  The Lahontan Regional Water Board adheres to the State Water Board’s recommended and upper TDS (SMCL of 500 mg/L and 1,000 mg/L, respectively (CA Water Boards, 2018; CA Regional Water Quality Control Board Lahontan Region, 2016).

Within the IWVGB, groundwater moves from the Mountains toward the China Lake playa, through coarse-grained alluvial deposits into fine-grained lacustrine deposits. This groundwater movement can cause dissolution of evaporites (caused by high evaporation rates), resulting in high TDS concentrations (TriEcoTt, 2013; Berenbrock and Schroeder, 1994).  Increased pumping can exacerbate the process described above causing ions to be leached from clay and lacustrine deposits resulting in increased TDS concentrations.  TDS trends for a number of wells sampled throughout the basin are shown in Figure 3-11.  TDS samples indicate concentrations have increased over time in areas where high rates of pumping have occurred.  The most recent TDS concentrations for wells sampled in the IWVGB are shown in Figure 3-12.  TDS concentrations in areas of the IWVGB are indicative of groundwater degradation.  Lab results for a number of wells sampled in the Navy/China Lake and northwestern areas show TDS concentrations considerably above the SMCL (ranging from 1,001 mg/L to >5,000 mg/L). IWV TDS data are provided in Appendix 3-C.

#### Arsenic

In semi-arid and arid groundwater basins, groundwater recharge is limited due to low precipitation and the residence time of the groundwater in the basin is high. The long residence time of the groundwater in the basin allows for more interaction between the groundwater and the minerals that comprise the aquifer matrix material. With time, arsenic, which is naturally occurring, desorbs from sediments and enters the groundwater.

Historically, a number of wells sampled within the IWVGB have shown arsenic concentrations above California’s current arsenic MCL (0.010 mg/L).  Existing arsenic data for IWV come from wells sampled in the Navy/China Lake area as most data reported were prepared for or in cooperation with the Department of the Navy (TriEcoTt, 2013; Tetra Tech EM Inc., 2003; Houghton HydroGeo-Logic, 1996; Bureau of Reclamation (BOR), 1993; Berenbrock, 1987).  Arsenic trends in four wells that have been sampled numerous times in the past are shown in Figure 3-13.  Similar to trends in TDS concentrations, arsenic concentrations appear to correlate with lacustrine/playa sediments in the Navy/China Lake area. In this area, the majority of samples collected had arsenic concentrations above the drinking water MCL (TriEcoTt, 2013; Tetra Tech EM Inc., 2003; Houghton HydroGeo-Logic, 1996; Bureau of Reclamation (BOR), 1993; Berenbrock, 1987). IWV arsenic data are provided in Appendix 3-E.

### Land Subsidence Conditions

Land subsidence can occur when long-term groundwater extractions result in the lowering of the groundwater table.  This can induce stress in the overlying aquifer matrix, causing the collapse of pore space within the matrix. Land subsidence can be either reversible (elastic), or irreversible (inelastic), depending on the soil characteristics of the aquifer.  Given the magnitude of current pumping in the IWVGB and the state of critical overdraft, land subsidence has the potential to occur due to the chronic lowering of groundwater levels. Testing facilities on NAWS China Lake are finely calibrated and thus would be particularly susceptible to negative effects due to land subsidence. DRI (McGraw et al, 2016) investigated land subsidence in the IWVGB using two methods:

- Direct measurement of land subsidence through Interferometric Synthetic Aperture Radar (InSAR)

- Simulation of land subsidence due to groundwater extractions continuing at 2016 rates using MODFLOW flow model

According to Katzenstein (2013), InSAR results indicate a subsidence rate of approximately 0.10 to 0.11 inches per year between 1992 and 2010.  It is unknown if the elevation changes observed with the InSAR data are due to faulting or land subsidence caused by groundwater extractions (McGraw et al, 2016). Model simulation results over-predicted land subsidence rates compared to the InSAR measurements; however, these differences could be attributed to model uncertainties as opposed to effects of sustained overdraft (McGraw et al, 2016).

Due to observation of land subsidence occurring in the IWVGB, and due to the limited knowledge of the causes, further land subsidence analysis was conducted by DRI for the GSP. This analysis is discussed in Section 4.2.8.

### Identification of Interconnected Surface Water Systems

As discussed in previously in Section 3.3.4.2, Little Lake, located in Rose Valley is the only significant interconnected surface water system that interacts with groundwater in the IWVGB, with surface water in Little Lake infiltrating into the groundwater basin. Streams in the valley are typically ephemeral and the majority of recharge occurs as mountain front recharge. Additionally, there are multiple natural springs in the IWV (see Figure 3-10). One spring located near Highway 14 is used as the water supply source for a lodge and brewery.

### Identification of Groundwater-Dependent Ecosystems (GDEs)

SGMA defines GDEs as “ecological communities of species that depend on groundwater emerging from aquifers or on groundwater occurring near the ground surface” (GSP, 2016). Groundwater is critical to sustaining springs, wetlands, and perennial flow (baseflow) in streams as well as to sustaining vegetation such as phreatophytes that directly tap groundwater.  Mapping of DWR’s Natural Communities Commonly Associated with Groundwater (NCCAG) dataset indicates the vast majority of GDEs within the IWV are located on NAWS China Lake.  Smaller and scattered communities of GDEs are present in the canyons along the Sierra Nevada, in the El Paso area, and in the southwest region of the IWV. A map of the different vegetative species comprising the GDEs within the IWV is shown in Figure 3-14. A field visit in November 1, 2018 verified the significant presence of GDEs in the China Lake Playa region on the NAWS China Lake. The GDEs located in the mountain canyons are not considered vulnerable due to mountain block recharge. Likewise, GDEs located in the El Paso region are not considered vulnerable due to steady groundwater levels. However, the GDES located in the valley floor, including those near the China Lake Playa, are likely more vulnerable due to chronic lowering of groundwater levels.  Currently, NAWS China Lake does not monitor desert vegetation on the base.

Due to the identification of GDEs in the IWV, further analysis was conducted to understand the potential undesirable effects to GDEs caused by the chronic lowering and groundwater levels and to inform the selection of sustainable management criteria. See Section 4.2.6.

## Numerical Groundwater Model

### Initial Model Development

The groundwater flow model of the IWV, used to evaluate SGMA compliance, was developed on a foundation of decades of hydrogeologic investigations of the basin. The first numerical model of the valley’s groundwater focused on the Ridgecrest and Inyokern areas and used a simplified aquifer geometry (Bloyd and Robson, 1971). Berenbrock and Martin (1991) used a more sophisticated representation of aquifer geometry in a subsequent model, but the system was simulated in a quasi-three-dimensional manner. These models provide a comprehensive catalog of hydraulic properties, though basin recharge, playa discharge, and predevelopment groundwater use are outdated based on more recent data. The U.S. Bureau of Reclamation (1993) updated the conceptual model of the basin and increased knowledge of the hydrogeology of IWV through the drilling, testing, and analysis of 10 deep wells on the western side of the valley. Additional drilling and testing are reported by Tetra Tech EM, Inc. (2003a, b), including extensive data and analysis regarding groundwater quality and isotopic composition.

The current groundwater flow model can be traced to the Brown and Caldwell model published in 2009. The recharge and ET were revised again for this 2009 model, and other conceptual model features were added. These updates include a fine-grained sediment plug in the western valley, a gravel zone, and a high gradient zone between the El Paso Subbasin and IWV. The Brown and Caldwell model successfully replicated some of the regional hydrogeologic features and was used to explore overdraft conditions. The major limitation of the 2009 model is that it cannot accurately reproduce drawdown rates. The model underpredicts drawdown within the city of Ridgecrest, but overpredicts drawdown northeast of Ridgecrest, indicating a spatial bias. The model errors also generally increase over time. An update of the Brown and Caldwell model occurred in 2011 with Layne Hydrologic (2011) refining the grid spacing, changing the solver configuration, and modifying initial conditions and pumping rates.

In 2016, DRI developed a groundwater flow model (McGraw et al., 2016) for the U.S. Navy to support their planning for the NAWS China Lake in response to declining groundwater levels and concerns about water quality degradation and subsidence. Starting with the Brown and Caldwell (2009) platform, mountain block recharge and playa evaporation rates were refined, the grid resolution was increased in the horizontal and vertical directions, model layering was refined to better represent aquifer units, boundary conditions were modified to allow flow to Salt Wells Valley, and a pilot-point hydraulic parameterization was performed to improve calibration to water levels and, importantly, to drawdown rates. A solute transport model and subsidence model were linked to the flow model to allow simulations of the impact of pumping on groundwater quality and land subsidence. Alternative conceptual models were analyzed, as were alternative future conditions. During 2017, DRI revised the model for the Navy by incorporating regional faults as groundwater barriers (Garner et al., 2017). This change corrected under-prediction of water levels in El Paso Valley by including the informally named EPF in the southwestern sector of the valley. The Little Lake Fault Zone was similarly added as a horizontal flow barrier trending northwesterly across the middle of IWV.

These DRI models (McGraw et al., 2016 and Garner et al., 2017) are the platform used for the SGMA simulations of baseline conditions and management scenarios. (See Section 3.5.2.1 for a discussion on the model review for suitability for the GSP). Model assumptions, construction, and performance are detailed in Appendix 3-F.  The GSP modeling effort provides tools necessary for estimating the groundwater aquifer’s hydrologic water budget, identifying data gaps, assessing groundwater level and quality trends, determining sustainability criteria, and evaluating different strategies to provide long-term sustainable groundwater management for the IWVGB.  The model also provides ongoing analysis and support as needed for the annual reports and periodic evaluations that will be required for submittal to DWR.

### Flow Model Review and Recalibration

#### Model Review

The IWVGA reviewed the DRI groundwater flow model to assess suitability for the GSP. A peer review was conducted by technical staff of the Water Resources Manager and approved by the IWV TAC.  This review included evaluating model documentation (McGraw et al., 2016; Garner et al., 2017), meetings with DRI modelers and Navy personnel, a hydrogeologic site visit, and assessment of model output files.  The guidelines used to evaluate the model included GSP Emergency Regulations (GSP, 2016), DWR best management practices for modeling (DWR, 2016c), and U.S. Geological Survey’s recommendations for evaluating groundwater models (Reilly and Harbaugh, 2004).  The DRI groundwater flow model was reviewed to 1) evaluate its efficacy in describing the groundwater basin structure, hydrogeologic characteristics, and inflows/outflows, and 2) determine its suitability for supporting the implementation of the IWVGA’s GSP through 2040.

The Water Resources Manager reviewed existing hydrogeologic reports and monitoring data to understand the elements of the conceptual model that were being simulated by the numerical model.  A groundwater model review checklist was developed based on the USGS guidelines (Reilley and Harbaugh, 2004), GSP Emergency Regulations (GSP, 2016), and DWR modeling best management practices (DWR, 2016c).

The model review evaluated the hydrogeologic representation of the available historical data and conceptual model, model construction, boundary conditions, aquifer properties, and fluxes (recharge, pumping, ET). The model output file was assessed by technical staff of the Water Resources Manager for water budget consistency and numerical convergence criteria.  Steady state and transient model calibration were evaluated based on the hydrogeologic conceptual basin model.

This review prompted additional changes to the flow model and a full recalibration. Changes to the flow model include incorporating additional hydraulic conductivity data, revision of the conceptual model in several areas, and sensitivity analyses for recharge, hydraulic conductivity, specific storage and specific yield. The conceptual model changes include representation of a zone of low hydraulic conductivity in the northern Brown Road area, extension of the zone of low hydraulic conductivity playa sediments southward, and increased vertical variability in hydraulic properties. The transport model was also updated to a revised salinity database and revised boundary conditions for the salinity of recharge.

#### Model Calibration

The IWV SGMA model was calibrated in three phases. The general approach has been to adjust the values of selected hydrogeologic parameters using manual and automated calibration processes until the model’s simulated results are consistent with observed historic trends in IWV and the El Paso sub-basin. The flow model was calibrated in two stages, steady state and transient, with comparisons made to observed water levels and water budgets in both cases. The calibration methods and results were reviewed and approved by the IWV TAC and its Model Ad-Hoc Committee during a series of meetings held in 2018 and 2019.

The steady-state groundwater flow model represents hydrologic conditions before large-scale groundwater pumping began in 1921. This model is calibrated to steady-state water levels measured in 132 wells in IWV in 1920 and four wells in the El Paso sub-basin. Pre-development water levels are not available in El Paso sub-basin, so recent stable water levels are used instead. During calibration, the values of two model parameters were varied: (1) horizontal hydraulic conductivity of the six model layers and (2) hydraulic characteristic (fault transmissivity divided by barrier width) of the two major faults included in the model. These parameters were chosen because in general they have significant effects on simulated water levels and because measurements of their values in IWV are limited and therefore are considered more uncertain than other parameters. Ranges of hydraulic conductivity were developed for the four primary subsurface hydrogeologic units in the valley by Brown and Caldwell (2009). The ranges of the parameters used for calibration and their calibrated values are described in Appendix 3-F. The results of calibration show that the steady-state flow model provides a good simulation of observed water levels in IWV (Figure 3-15). The mean absolute error (MAE) between simulated and observed water levels is 6 feet. The relative error in water levels, which is the MAE divided by the range in observed water levels, is 0.84 percent, which is well below the 10-percent threshold that is generally considered an acceptable maximum relative error for predictive models.

Groundwater conditions during the period 1921 through 2016 are simulated by the transient flow model. This model is calibrated to water budget terms and historic water-level trends and altitudes observed in 36 wells (Figure 3-16) by adjusting values of the transient storage parameters specific storage and specific yield. The values of the hydraulic conductivity and fault hydraulic characteristic parameters determined during calibration of the steady-state model were adopted unchanged in the transient model. Model results were found to be insensitive to specific storage so this parameter was assigned a constant value of 3x10-7 ft-1 for all confined model layers. The transient model uses observed rates of water-level drawdown that resulted from groundwater pumping as a calibration metric to supplement the MAE of water level altitudes. This approach was taken because the rate of drawdown is the critical factor for simulating the effect of overdraft conditions. The water-level altitude is generally controlled by hydraulic conductivity and recharge, parameters that were determined in the steady-state model, while drawdown rates are strongly affected by storage parameters. This approach minimizes errors in the transient calibration that might result from the model attempting to resolve offsets in water level altitudes simulated by the steady-state model. A robust regression slope-fitting approach was used to remove observation outliers and compute the differences in slope of the simulated and observed water-level trends at the 36 observation locations. An example of the slope-fitting approach is shown in Figure 3-17 The results of both calibration metrics as they relate to specific yield values are shown in Figure 3-18. The optimal solution using drawdown slope as the metric is obtained for a specific yield value of 0.225, whereas an optimum specific yield of 0.25 results when using MAE of water levels as the metric. Although these values are very close in magnitude, the value of 0.225 was selected because the drawdown-slope approach more accurately represents how the model is applied for predictive pumping scenarios. The transient model was also calibrated to water budget terms and demonstrated excellent agreement with measured ET rates.

The transport model uses concentrations of TDS as a surrogate for groundwater salinity and is based on initial TDS concentrations developed from a TDS database compiled from over 400 wells over a period of 70 years. The most-recent TDS concentration for each sample location (if multiple values were available) were supplemented by control points in regions of sparse data to provide the framework for interpolation of a spatial distribution of initial TDS to all model cells. A quantitative calibration of the transport model was not performed because the initial TDS distribution represents available measurements that span many decades, thus a historic simulation period could not be developed. Instead, a qualitative calibration compares forecasts of TDS trends simulated by the transport model to general historic trends and spatial distributions represented by the conceptual model of salinity.

### Baseline Conditions

[Not yet completed]

## Existing Monitoring Network and Evaluation

### Existing Monitoring Network

[Not yet completed]

### Monitoring Network Assessment and Data Gap Evaluation

[Not yet completed]

#### Stream Gages

[Not yet completed]

#### Weather Stations

[Not yet completed]

#### Water Quality Sampling and Stable Isotope Sampling

[Not yet completed]

#### Monitoring Wells

[Not yet completed]

#### Aquifer Tests

[Not yet completed]

## References

Berenbrock, C., 1987. Ground-water data for Indian Wells Valley, Kern, Inyo, and San Bernardino Counties, California, 197-84. U.S. Geological Survey Open File Report 86-315. Prepared in cooperation with the Indian Wells Valley Water District and the US Department of the Navy, China Lake Naval Weapon Center.

Berenbrock, Charles and Peter Martin, 1991.  The Ground-Water Flow System in Indian Wells Valley, Kern, Inyo, and San Bernardino Counties, California.  USGS Water-Resources Investigations Report 89-4191.  Prepared in cooperation with Indian Wells Valley Water District and the U.S. Deportment of the Navy, China Lake Naval Weapons Center.  87pp.

Berenbrock, C., and Schroeder, R.A., 1994. Ground-water flow and quality, and geochemical processes, in Indian Wells Valley, Kern, Inyo, and San Bernardino Counties, California, 1987-88, Water-Resources Investigations Report 93-4003. United States Geological Survey. Prepared in cooperation with the Indian Wells Valley Water District and the U.S. Department of the Navy, China Lake Naval Weapons Center.

Bloyd, R.M. and S.G. Robson, 1971.  Mathematical Ground-Water Model of Indian Wells Valley, California.  USGS Open-File Report 72-41.  Prepared in cooperation with the Indian Wells Valley County Water District and the Department of the Navy.  39pp.

Brown and Caldwell, 2009.  Indian Wells Valley Basin Groundwater Flow Model and Hydrogeologic Study.  Prepared for IWVWD.  March 2009.

California Department of Water Resources (DWR), 1980. Groundwater Basins in California. Bulletin 118‐80. Sacramento, CA.

____, 2016a.  Groundwater Sustainability Plan (GSP) Emergency Regulations Guide.  Sustainable Groundwater Management Program, July, 2016.

____, 2016b.  Hydrogeologic Conceptual Model Best Management Practice.  Sustainable Groundwater Management Program BMP, 42pp.  December, 2016.

____, 2016c.  Modeling Best Management Practice.  Sustainable Groundwater Management Program BMP, 42pp.  December, 2016.

____, 2016d.  Water Budget Best Management Practice.  Sustainable Groundwater Management Program BMP, 53pp.  December, 2016.

California Regional Water Quality Control Board Lahontan Region, 2016. Water quality control plan for the Lahontan Region.  January 2016.

California Water Boards, 2018. Secondary drinking water standards. Accessed at: https://www.waterboards.ca.gov/drinking_water/certlic/drinkingwater/documents/ddw_secondary_standards.pdf, April 15, 2019.

_____, 2019. Contaminants in drinking water. Accessed at: https://www.waterboards.ca.gov/drinking_water/certlic/drinkingwater/Chemicalcontaminants.html, April 15, 2019.

Danskin, W.R., 1998. Evaluation of the hydrologic system and selected water management alternatives in the Owens Valley, California. USGS Water Supply Paper 2370‐H

Dokka, R.K., and Travis, C.J., 1990. The role of the Eastern California shear zone in accommodating Pacific–North American plate motion. Geophysical Research Letters 17, 1323–1326.

Dutcher L.C. and W.R. Moyle, 1973.  Geologic and Hydrologic Features of Indian Wells Valley, California. USGS Water Supply Paper 2007.  Prepared in cooperation with the California Department of Water Resources.

Epstein, B., G.M. Pohll, J. Huntington, and R.W.H. Carroll, 2010. Development and uncertainty analysis of an empirical recharge prediction model for Nevada’s desert basins. Nevada Water Resources Association. 5(1): 1-22

Farnsworth, R.K., Thompson, E.S., and Peck, E.L. 1982. Evaporation Atlas for the Contiguous 48 United States. National Oceanic and Atmospheric Administration, National Weather Service. NOAA Technical Report NWS 33.

Garner, C., Bacon, S., Pohll, G., and Chapman, J. 2017. Technical memorandum: Indian Wells Valley groundwater model update. Prepared by Desert Research Institute. November 17, 2017.

GSP §350 to §358.4 from CCR, 2016.  California Code of Regulations; Title 23. Waters; Division 2. Department of Water Resources; Chapter 1.5. Groundwater Management; Subchapter 2. Groundwater Sustainability Plans.

Harris, Glenn. 2013a, March 23. History of Water Use in the Indian Wells Valley, Part I. The Daily Independent, Retrieved from www.ridgecrestca.com.

_____, 2013b, September 11. History of Water Use in the Indian Wells Valley, Part II. The Daily Independent, Retrieved from www.ridgecrestca.com.

Haslebacher, T. C. 2018. Indian Wells Valley Stream Monitoring Report. February 04, 2018-March 09,2018 – Field Activities and Data Interpretation. April 3, 2018.

Hauksson, E., Hutton, K., Kanamori, H., Jones, L., Mori, J., Hough, S., and Roquemore, G., 1995. Preliminary report on the 1995 Ridgecrest earthquake sequence in eastern California. Seismological Research Letters 66, 54-60.

Hereford, R., Webb, R.H. and Longpre, C.I., 2004. Precipitation history of the Mojave Desert region, 1893-2001 (No. 117-03). United State Geological Survey.

Houghton, HydroGeo-Logic, 1996. Geohydrologic investigation report Naval Air Weapons Station, Indian Wells Valley, China Lake, California. Prepared for Naval Air Weapons Station Environmental Projects Office, China Lake, California. November 1996.

Katzenstein, K.W., 2013. Evaluating Potential Land Subsidence Induced by Groundwater Withdrawal from the Indian Wells Valley, CA Using InSAR. Geological Society of America Abstracts with Programs. Vol. 45, No. 7, p.775.

Kunkel, Fred and G.H. Chase, 1969.  Geology and Ground Water in Indian Wells Valley, California.  USGS Open-File Report 69-329.  Prepared in cooperation with the Naval Weapons Center, China Lake, California.  89pp.

Lahee, F.H. 1961. Field Geology. McGraw-Hill Book Company, Inc. New York.

Layne-Western Hydrologic, 2011. Simulated impacts of changes to IWVWD wells. Appendix G in ECORP Consulting Inc., Draft Environmental Impact Report Water Supply Improvement Project. Prepared for Indian Wells Valley Water District.

Lee, Charles H., 1912.  Groundwater Resources of Indian Wells Valley.  Report of the Conservation Commission of the State of California.

McGraw, D., Carroll, R., Pohll, G., Chapman, J., Bacon, S., and Jasoni, R. 2016. Groundwater Resource Sustainability: Modeling Evaluation for the Naval Air Weapons Station, China Lake, California. Report NAWCWD TP 8811.  Prepared by Desert Research Institute. Prepared for Naval Air Warfare Center Weapons Division, China Lake. September 2016.

Moyle, 1963. Data on Water Wells in Indian Wells Valley Area. Prepared for DWR Bulletin No. 91-9; prepared by United States Department of Interior Geological Survey.

Monastero, F.C., Walker, J.D., Katzenstein, A.M., and Sabin, A.F. 2002. Neogene evolution of the Indian Wells Valley, east-central California. Geological Society of America Memoir 195.

PRISM Climate Group, Oregon State University. 2012. 1981-2010 Annual Precipitation Climate Normals. 800m raster grid. http://www.prism.oregonstate.edu/normals/

Reilly, T.E. and A.W. Harbaugh, 2004.  Guidelines for Evaluating Ground-Water Flow Models. U.S. Geological Survey Scientific Investigations Report 2004-5038, 30pp.

Ribble, G.E. and Haslebacher, T. 2000. A Methodology for Estimating Water Yield of Sierra Watersheds Tributary to Indian Wells Valley. Prepared by Kern County Water Agency. February 2000.

Stoner, M., Bilhorn, T., Maas, J., and Dickey, S. 1995. Native Spring Investigation Data Report (No. NAWS-CL-TP-005). Naval Air Weapons Station China Lake CA.

Tetra Tech EM, Inc. (TtEMI), 2003a.  Draft Basinwide Hydrogeologic Characterization Summary Report. SAWS China Lake, California.  Prepared under CLEANII Contract for Department of the Navy. January 2003.

Tetra Tech EM, Inc. (TtEMI), 2003b.  AB 303 Grant; Groundwater Management in the Indian Wells Valley Basin, Ridgecrest, California.  Prepared for Eastern Kern County Resources Conservation District.  June 2003.

Thorn, P., Parker, T., and Halkjaer, M., 2019. Draft: Indian Wells Valley hydrogeologic conceptual model traditional HCM. Prepared for Indian Wells Valley Water District and the Brackish Groundwater Resources Study Group. March 2019.

Todd Engineers, 2014.  Indian Wells Valley Resource Opportunity Plan, Water Availability and Conservation Report.  Prepared for Kern County Planning and Community Development Department.  January 2014.

TriEcoTt, 2013. Technical justification of beneficial used changes for groundwater in Salt Wells Valley and shallow groundwater in Eastern Indian Wells Valley. Prepared by TriEcoTt, a joint venture of TriEco LLC and Tetra Tech EM Inc. Prepared for Department of the Navy. February 2013.

U.S. Bureau of Reclamation (USBR), 1993. Indian Wells Valley Groundwater Project: USBR Technical Report Volume II.  A cooperative effort among the BOR, IWVWD, North American Chemical Company, and NAWS.  December 1993. Prepared by USBR Lower Colorado Region.

United States Department of Agriculture (USDA) Natural Resources Conservation Service (NRCS), 1998. Dominant soil orders. Accessed at:  	 https://www.nrcs.usda.gov/Internet/FSE_MEDIA/stelprdb1237749.pdf, April 4, 2019.

United States Department of Agriculture (USDA) Natural Resources Conservation Service (NRCS), 2019.  The twelve orders of soil taxonomy. Accessed at:     	 https://www.nrcs.usda.gov/wps/portal/nrcs/detail/soils/survey/class/data/?cid=nrcs142p2_053588, April 4, 2019.

U.S. Geological Survey (USGS), National Geospatial Program. 2019. USGS National Hydrography Dataset Best Resolution (NHD) for Hydrologic Unit (HU) 4 - 1809 (published 20190401). Reston, VA. Accessed at: ftp://rockyftp.cr.usgs.gov/vdelivery/Datasets/Staged/Hydrography/NHD/HU4/HighResolution/GDB/NHD_H_1809_HU4_GDB.zip

Von Huene, R.E., 1960. Structural geology and gravimetry of Indian Wells Valley, southeastern California: California State University, Los Angeles, Ph.D. thesis, 138 p.

Warner, J.W., 1975. Ground-water quality in Indian Wells Valley, California: U.S. Geological Survey Water Resources Investigations Report 8-75, 59 p.

Wesnousky, S.G., 2005. Active faulting in the Walker Lane. Tectonics, 24, TC3009, doi:  10.1029/2004TC001645.

Western Regional Climate Center (WRCC). 2018a. 1971 – 2000 Climate Normals for Stations 041733 (China Lake NAF), 044278 (Inyokern), 049035 (Trona), 043710 (Haiwee), 047253 (Randsburg), and 044232 (Independence). Accessed at: https://wrcc.dri.edu/Climate/west_coop_summaries.php, December 5, 2018.

Western Regional Climate Center (WRCC). 2018b. Period of Record Monthly Climate Summary for Cooperative Stations 041733 (China Lake NAF), 044278 (Inyokern), 049035 (Trona), 043710 (Haiwee), 047253 (Randsburg), and 044232 (Independence). Accessed at: https://wrcc.dri.edu/Climate/west_coop_summaries.php, December 5, 2018.

______. 2018b. Remote Automated Weather Station (RAWS) Data for Five Mile, Blue Max, Indian Wells Canyon, Walker Pass, Bird Springs, and Laural Mountain Stations. Accessed at: https://raws.dri.edu/scaF.html, December 5, 2018.

Zbur, R.T., 1963. A geophysical investigation of Indian Wells Valley, California: U.S. Naval Ordnance Test Station, China Lake, California, NOTS Technical Publication 2795, 98 p.

Appendix 3-A lists a category of water users called “Private Wells”. “Private Wells” in this context also includes water use from mutual water companies and co-ops; therefore, the combined total of private domestic wells and mutual water companies and co-ops is 1,100 AF in 2015.